Home Automation Final Report

June , 2019

Project Members:

Shubam Pandit

Rachit Gupta

Ajay Singh

Shashvat Sharma

Government College Of Engineering & Technology Jammu

Computer Department

Mentor: Prof. Neeraj Dubey

Home Automation

Table Of Contents                                                     Page

- Abstract

- Acknowledgement

- Objective

- Description Of Project

- Requirement analysis

- Description Of Hardware Used

- Description Of Software Used

- Design And Implementation

- Pros And Cons

- Application

- Future Development

- Conclusion

- Reference

Abstract

Automation is the main application of each and every technology now a days. Cloud automation helps to reduce the manual efforts associated with provisioning and managing computing power. Similarly Iot automation helps to reduce manual work of actuating appliances or sensing by making these devices internet-enabled.

Home automation refers to the ability to automate manual tasks that we perform at home. It can be realized in many different ways. The home automation system discussed in this paper makes use of IOT and cloud automation both.

By applying home automation to our homes we are turning them to smart homes. These homes use a centralized control system to control the whole house. Things like controlling appliances, security alarm systems, fire alarm systems, temperature control etc can be achieved from a single tap on your smartphone. Above all it’s a great help for elderly or physically handicapped people at home.

Acknowledgement

First and foremost, our team would like to take this opportunity to thank our mentor and guide Mr. Neeraj Dubey for his guidance and constant support on this project. We are really thankful of each an every possible help which was provided to us by the staff of our computer department. This project could not be possible without the brainstorming and hard work done by all the team members in this project. Thanks to everyone related to this project.

Objective

The objective of this project is to implement a low cost, reliable and scalable home automation system that can be used to remotely control different aspects of the house. The different aspects include controlling appliances, intruder alarm system, fire alarm system, temperature control, garage shutter control etc.

To realize these objectives we aim to design this system using arduino boards, wifi modules and a cloud storage.

To interact with this system remotely we provide android app for smartphone users and a website for other platform users.

Description of project

Proposed System

We propose a basic home automation system with these features:

Automated lights

One can easily control the lights of the room with the help of the mobile app connected to his home wifi. These lights also have the capability of automatically turn on or off depending upon whether a person is inside or outside the room.

Automated Garage door

Turns  the garage open or close with the help of a single click on the mobile or web app.

Temperature and information display

A led display to display important information as well as temprature and humidity so that person is informed everytime.

Intrusion detection

Once a person is sleeping or not at home, the system has a capability to detect any intrusion in that persons home and notify that with an alarm.

Fire alarm

If the house ever catches fire, the system will notify the user and set an alarm on which can help notify others also those  can help us.

Proposed model diagram

Working

The model works in a master slave operation in which node mcu is a master and arduino uno controller is a slave. The mcu requests the arduino for all the sensors data in order to update them in cloud. Then the mcu connects to the cloud and update the data in the cloud's database. When a user presses a button in the app to do anything, the value is changed inside the cloud. And these changed values are reflected back to the node mcu, which is constantly listening to the cloud. Then the nodemcu uses the changed values to order the arduino to do the required change. For Example when i push a led button off in the app the value of led stored inside the cloud is changed to 0 ie OFF. So nodemcu notices the change because it is contantly listening to the cloud for any change. When nodemcu notices that led variable is 0 then it order the slave arduino to turn off the light corresponding to led variable. This whole change is reflected in terms of a state which means the arduino is constantly monitoring a state of the current house which contain all the information about all the sensors and light etc. The mcu is monitoring the state of the cloud and the database. So any change in either of the state will always be represented. The medium used to represent these states is JSON. It has the capability to monitor each state effectively.The whole information processing part is done on the nodemcu that is why it is a master in this context. The job of the uno controller is to only transfer the current state and update the state after getting order from the master ie Nodemcu.

Requirement Analysis

For this type of project there are two types of requirements

- Hardware Requirements

- Software Requirements

Let us first talk about the hardware requirements

Hardware Requirement

First and the foremost  requirement is that of an infrastructure or a home for performing automation.

A small model house is used as the infrastrcture to perform automation upon.

Multiple electronic components are used for building electronic circuits

Table 1: Specifications of electronic components used in to design the proposed system.

| Components | Specifications |
| --- | --- |
| 1.Arduino Uno | 22 pins, operating voltage 6-20V |
| 2. Node mcu | Esp8266 cpu |
| 3. LED’S | 5V |
| 4 LCD | 16 X 2 DISPLAY |

| 5. PIR Sensor | Voltage: DC 3-5V, Range 2-30cm |
| --- | --- |
| 6. Step up Motor | 24BYJ48 Stepper Motor |
| 7. Infrared Flame Detection Sensor Module | 3 pin 5 volt |
| 8. Buzzer | 5v |
| 9. Jump WIRE | M-M , M-F, F-F |
| 10. Breadboard | Standard |

Software Requirements

Languages used:

For arduino prorammimg

For Android app

For Website

Translators used:

For arduino programming

For Android app

For Website

Cloud service/ Database used:

Description of Hardware used

Arduino Uno

Arduino Uno is a microcontroller board based on the ATmega328P (datasheet). It has 14 digital input/output pins (of which 6 can be used as PWM outputs), 6 analog inputs, a 16 MHz quartz crystal, a USB connection, a power jack, an ICSP header and a reset button. It contains everything needed to support the microcontroller; simply connect it to a computer with a USB cable or power it with a AC-to-DC adapter or battery to get started.. You can tinker with your UNO without worring too much about doing something wrong, worst case scenario you can replace the chip for a few dollars and start over again."Uno" means one in Italian and was chosen to mark the release of Arduino Software (IDE) 1.0. The Uno board and version 1.0 of Arduino Software (IDE) were the reference versions of Arduino, now evolved to newer releases. The Uno board is the first in a series of USB Arduino boards, and the reference model for the Arduino platform; for an extensive list of current, past or outdated boards see the Arduino index of boards.

Tech specs

Programming

The Arduino Uno can be programmed with the (Arduino Software (IDE)). Select "Arduino/Genuino Uno from the Tools > Board menu (according to the microcontroller on your board). For details, see the reference and tutorials.The ATmega328 on the Arduino Uno comes preprogrammed with a bootloader that allows you to upload new code to it without the use of an external hardware programmer. It communicates using the original STK500 protocol (reference, C header files).

NodeMcu

NodeMCU is an open source Lua based firmware for the ESP8266 WiFi SOC from Espressif and uses an on-module flash-based SPIFFS file system. NodeMCU is implemented in C and is layered on the Espressif NON-OS SDK.

The firmware was initially developed as is a companion project to the popular ESP8266-based NodeMCU development modules, but the project is now community-supported, and the firmware can now be run on any ESP module.

It has following features

- Built-in Micro-USB, with flash and reset switches, easy to program

- Full I/O port and Wireless 802.11 supported, direct download no need to reset

- Arduino compatible, works great with the latest Arduino IDE/Mongoose IoT/Micropython

LED’s

A light-emitting diode (LED) is a semiconductor light source that emits light when current flows through it. Electrons in the semiconductor recombine with electron holes, releasing energy in the form of photons. This effect is called electroluminescence.The color of the light (corresponding to the energy of the photons) is determined by the energy required for electrons to cross the band gap of the semiconductor. White light is obtained by using multiple semiconductors or a layer of light-emitting phosphor on the semiconductor device.

Appearing as practical electronic components in 1962, the earliest LEDs emitted low-intensity infrared light. Infrared LEDs are used in remote-control circuits, such as those used with a wide variety of consumer electronics. The first visible-light LEDs were of low intensity and limited to red. Modern LEDs are available across the visible, ultraviolet, and infrared wavelengths, with high light output.

Early LEDs were often used as indicator lamps, replacing small incandescent bulbs, and in seven-segment displays. Recent developments have produced white-light LEDs suitable for room lighting. LEDs have led to new displays and sensors, while their high switching rates are useful in advanced communications technology.

LEDs have many advantages over incandescent light sources, including lower energy consumption, longer lifetime, improved physical robustness, smaller size, and faster switching. Light-emitting diodes are used in applications as diverse as aviation lighting, automotive headlamps, advertising, general lighting, traffic signals, camera flashes, lighted wallpaper and medical devices.

Unlike a laser, the color of light emitted from an LED is neither coherent nor monochromatic, but the spectrum is narrow with respect to human vision, and functionally monochromatic.

LCD Module

We come across LCD displays everywhere around us. Computers, calculators, television sets, mobile phones, digital watches use some kind of display to display the time. An LCD is an electronic display module which uses liquid crystal to produce a visible image. The 16×2 LCD display is a very basic module commonly used in DIYs and circuits. The 16×2 translates o a display 16 characters per line in 2 such lines. In this LCD each character is displayed in a 5×7 pixel matrix.

PIR SENSOR

A passive infrared sensor (PIR sensor) is an electronic sensor that measures infrared (IR) light radiating from objects in its field of view. They are most often used in PIR-based motion detectors. PIR sensors are commonly used in security alarms and automatic lighting applications. PIR sensors detect general movement, but do not give information on who or what moved. For that purpose, an active IR sensor is required.

PIR sensors are commonly called simply "PIR", or sometimes "PID", for "passive infrared detector". The term passive refers to the fact that PIR devices do not radiate energy for detection purposes. They work entirely by detecting infrared radiation (radiant heat) emitted by or reflected from objects.

How PIRs Work

by lady ada

PIR sensors are more complicated than many of the other sensors explained in these tutorials (like photocells, FSRs and tilt switches) because there are multiple variables that affect the sensors input and output. To begin explaining how a basic sensor works, we'll use this rather nice diagram

The PIR sensor itself has two slots in it, each slot is made of a special material that is sensitive to IR. The lens used here is not really doing much and so we see that the two slots can 'see' out past some distance (basically the sensitivity of the sensor). When the sensor is idle, both slots detect the same amount of IR, the ambient amount radiated from the room or walls or outdoors. When a warm body like a human or animal passes by, it first intercepts one half of the PIR sensor, which causes a positive differential change between the two halves. When the warm body leaves the sensing area, the reverse happens, whereby the sensor generates a negative differential change. These change pulses are what is detected.

The PIR Sensor

The IR sensor itself is housed in a hermetically sealed metal can to improve noise/temperature/humidity immunity. There is a window made of IR-transmissive material (typically coated silicon since that is very easy to come by) that protects the sensing element. Behind the window are the two balanced sensors.

Steper motor

A Stepper Motor or a step motor is a brushless, synchronous motor which divides a full rotation into a number of steps. Unlike a brushless DC motor which rotates continuously when a fixed DC voltage is applied to it, a step motor rotates in discrete step angles. The Stepper Motors therefore are manufactured with steps per revolution of 12, 24, 72, 144, 180, and 200, resulting in stepping angles of 30, 15, 5, 2.5, 2, and 1.8 degrees per step. The stepper motor can be controlled with or without feedback.

Infrared flame detection sensor

A flame detector is a sensor designed to detect and respond to the presence of a flame or fire, allowing flame detection. Responses to a detected flame depend on the installation, but can include sounding an alarm, deactivating a fuel line (such as a propane or a natural gas line), and activating a fire suppression system. When used in applications such as industrial furnaces, their role is to provide confirmation that the furnace is working properly; in these cases they take no direct action beyond notifying the operator or control system. A flame detector can often respond faster and more accurately than a smoke or heat detector due to the mechanisms it uses to detect the flame.

Buzzer

A buzzer or beeper is an audio signalling device,which may be mechanical, electromechanical, or piezoelectric (piezo for short). Typical uses of buzzers and beepers include alarm devices, timers, and confirmation of user input such as a mouse click or keystroke.

Jump wires

A jump wire (also known as jumper wire, or jumper) is an electrical wire, or group of them in a cable, with a connector or pin at each end (or sometimes without them – simply "tinned"), which is normally used to interconnect the components of a breadboard or other prototype or test circuit, internally or with other equipment or components, without soldering.Individual jump wires are fitted by inserting their "end connectors" into the slots provided in a breadboard, the header connector of a circuit board, or a piece of test equipment.

Bread Board

A breadboard is a construction base for prototyping of electronics. Originally the word referred to a literal bread board, a polished piece of wood used for slicing bread.

In the 1970s the solderless breadboard (a.k.a. plugboard, a terminal array board) became available and nowadays the term "breadboard" is commonly used to refer to these.

Because the solderless breadboard does not require soldering, it is reusable. This makes it easy to use for creating temporary prototypes and experimenting with circuit design. For this reason, solderless breadboards are also popular with students and in technological education. Older breadboard types did not have this property. A stripboard (Veroboard) and similar prototyping printed circuit boards, which are used to build semi-permanent soldered prototypes or one-offs, cannot easily be reused. A variety of electronic systems may be prototyped by using breadboards, from small analog and digital circuits to complete central processing units (CPUs).

Description of Software used

Design and implementation

Model of the house (Top view)

Cricuit Diagam

Pros and cons

Pros

Save Money on Utilities

Utilities can amount to several hundred dollars per month. Home automation can turn off lights or lower the thermostat automatically when you aren’t using them and easily lower your utility bills by 10% to 25%

Increased Home Safety

Many accidents happen in the home because of poor lighting. Home automation can automatically turn lights on in closets, stairways, and other dark places when you enter and decrease the chance of accidentally tripping or running into things

Home Security

Although home security is a priority for everyone, high installation cost or monthly monitoring charges make security systems cost prohibitive for many homeowners. Home automation provides an inexpensive solution to home security.

Good for the Environment

In a time when we are all becoming more environmentally aware, home automation provides a good solution to help preserve our natural resources. Home automation products can reduce power consumption and automatically turn off lights and appliances when they aren’t in use.

Peace of Mind

Never again worry about your home while you’re away. Using home video camerasand an internet connection, you can check on the status of your home or kids from anywhere in the world using a PC or web-enabled phone.

Learning Experience for Children

Technology is here to stay and the more your children learn about upcoming technology the better prepared they are for the future. Turn your home into a classroom, as your home automation projects become a learning experience for your children.

Cons

Applications

Future development

Conclusion

Referenes